I want to refactor this version to a new version. The new version talks to 2 alicat devices. Alicat "A" and Alicat "B". Talk to them by changing the command sting to A or B. For example to as pressure on A the command is AS1.0 for B it is BS1.0 terminate by CR

Test steps are as follows:
Read ini file to get test pressures, duration and other test parameters

Flow Test

Send Release valve command to A (A

Set “A” to the A_FLOW_TEST PRESSURE (AS#.#/r)

Set “B” to  B_FLOW_TEST_PRESSURE (BS#.#/r)

Wait until the pressure stabilizes

Read and save mass flow for both A and B for FLOW_SAMPLE_TIME found in ini file Default 5s

Pressure Decay Test

Set B pressure to B_DECAY_TEST_PRESSURE (Default is 0 to close valve) BS0/r

Wait for pressure in A to stabilize and send close valve command (AHC/r)

Constantly read and record the pressure from A for PRESSURE_SAMPLE_TIME from ini default 20 seconds

Data is read at READ_RATE from ini file Default is 1s

Like the original program all data is stored in a CSV file. First sheet is test parameters and second page contains measurement data